Heat Engines

Now we will look at an important application of Thermodynamics – namely heat engines and refrigerators.  These advances made the industrial revolution possible, and Florida habitable.  The most conspicuous example of a heat engine is the one in your car, but they are used everywhere, including power generating stations like coal plants, nuclear power plants, etc.  Refrigerators are ubiquitous of course too.  There is one in your home, as well as one (AC) in your car.

Now let’s consider a few examples pertaining to heat engines.  Heat engines are machines that are designed to absorb heat and convert some of it to work.  Let’s consider an example.  Suppose we want to make a machine that raises a weight, m = 1000 kg, from the ground to the top of a building.  One way to do this is via the following contraption,

Its basically a fishing pole but with a heat engine, instead of you, connected to the ‘reel’.  The piston moves up and down rotating the wheel clockwise.  And as the wheel rotates clockwise the line wraps around it, pulling the weight up in the process.  If we ignore the line, this is also how the pistons in your car rotate the wheels.

In order to get the wheel to rotate, we would transfer heat to the gas by lighting a flame under it perhaps.  This would cause the gas to expand, and the piston to move upward, making the wheel rotate CW.  Then on the way back down, we would to take heat out of the gas (perhaps by putting it in an ice-water bath) which would cause the gas to contract and the piston to move downward, continuing the CW rotation of the wheel.  This is the basic idea.  But there are good ways to do this, and better ways to do this.  By changing how much heat we add and when we do it, we can increase the efficiency of our engine.  Let’s look at a few different possibilities.

Method 1: Isobaric cycle

Now suppose our piston contains n = 1.2 mol of air, which we’ll approximate as a diatomic gas.

(1) → (2)  Isobaric expansion:  Let’s say it starts off at volume V1 = 0.001m3, and pressure P1 = 6Mpa.  FYI, according to the equation of state, PV = NkT, the temperature of the gas must be T = 600K.  Then we let the piston go and start heating the gas underneath at a constant rate so that it steadily pushes the piston up to the top of the chamber with constant  pressure P1. We can make sure it pushes the piston up at constant pressure by controlling the rate at which we add heat to the gas (which we can do by say adjusting how closely we hold the match).  This will take us to situation (2), where the pressure of the gas is P2 = 6Mpa, and its volume is V2 = 0.01m3.  And from the equation of state, we can determine the temperature then to be T2 = 6000 K.

(2) → (3) Isometric cooling:  Now that the piston is at the top of the chamber, the wheel’s angular momentum will make it tend to keep going CW, which will tend to make the piston go back down.  To let this happen, we have to lower the pressure of the gas.  So while the piston is at the top we would cool the gas (maybe with some ice cubes – there are better ways to do this of course but you get the idea) so that it pressure drops to say P3 = 1 Mpa.  Since the piston is still near the top the volume will be V3 = 0.01 m3.  According to the equation of state, the temperature will now be: T3 = 1000 K.

(3) → (4)  Isobaric contraction.  Now we allow the piston to drop at a constant pressure P3 = 1 Mpa, until it returns to its starting position.  There we will have V4 = 0.001m3, P4 = 1 Mpa, and according to the equation of state, T4 = 300 K.

(4) → (1)  Isometric heating:  Finally, to prepare the gas to push the piston back up, we need to increase its pressure.  So, while the piston is at the bottom of the cycle we will heat the gas up until its pressure returns to the starting value P1 = 6 Mpa.

This will complete the cycle, which we could repeat to continue raising the weight the rest of the way.  It is useful to draw this process on a so-called P-V diagram.  This would look like the following:

We start at (1).  Then we heated our gas at constant pressure to bring us to (2).  Then we cool the gas at constant volume to bring us to (3).  And then we cool the gas at constant pressure to bring us to (4).  Finally, we heat the gas at constant volume to bring us to (1).

Now let’s ask a few questions.  How much work does the machine accomplish?  How much heat do we have to put in the machine to make it operate?  This is relevant because the more heat we have to put in, the more expensive it would be to operate.  How much heat is output by the machine?  We would need to know this because we would need to design a cooling system capable of absorbing all that heat.  What is the change in energy of our gas from beginning to end of the cycle?  What is the total change in entropy of our apparatus during the cycle (assume that the temperature of our flame is 6000K, and that the temperature of our ice bath is 300K)?  What is the efficiency of the machine?  The efficiency is defined as the ratio of W to heat input Qin.  Obviously we’d like our machine to be as efficient as possible.

Net work done by gas

And now we can calculate the net work done by our gas.  The net work is the sum of the works done during each of the 4 processes.

Note that this is just the area inside the P-V curve.  This is always the case.  So we may also say, for a cyclic process where the gas ends and begins at the same point on the P-V curve:

In any event, this works turns out to be:

Since the weight has a mass m = 1000 kg, this means that we could raise it h = W/mg = 4.6 m every cycle.

Heat input

What is the heat that we have to input?  We input heat during processes (1) → (2), and during (4) → (1).  So the net heat input is:

The heat input during the first process can be determined.  From the 1st law it is:

We can determine the heat input during (4) → (1) using the 1st law of thermodynamics,

Therefore the net heat input is:

What is change in energy of our gas?

This is easy to answer.  The change in energy of our gas is:

But note that the temperature of our gas doesn’t change from beginning to end of cycle.  This is because at beginning and end, it has the same P and V.  And since PV = NkT,  that means it has the same T as well.  So ΔT = 0 → ΔE = 0.  This is always the case for a cyclic process.

What is the heat output?

From the 1st law, we have,

Let’s apply this to the entire cycle.  Then we must have,

What is the total change in entropy of our system?

Our system comprises the machine and the heat / cold sources that transfer heat to and from the machine.  This is a closed system since energy is transferred only between these objects.  And now we’d like to know the net change in entropy of this system.  Well,

The reasoning is this.  First, the net change in entropy of our gas is 0 because it goes through a cyclic process.  It begins and ends in the same state (T1, V1) and so it must have the same entropy at the end as it does in the beginning.  The ice bath increases in entropy because it accepts an amount of heat Qout = 174 kJ at a temperature of 300K (we make the standard simplifying assumption that the ice bath is so large that accepting that heat doesn’t change its temperature appreciably).  The fire decreases entropy because it gives an amount of heat Qin = 219 kJ away at temperature T = 6000K.

What is the efficiency of our machine?

The efficiency of our engine is defined as the net work done divided by the heat input.

and so this works out to be:

This means that 21% of the (heat) energy we put into the gas gets output as work.  That’s not too bad.  The human body by comparison is about 20% efficient in converting chemical potential energy from food to work.  But even still, is there any way to improve our efficiency?  There is actually.

Method 2: Isothermal cycle

The basic idea of the isothermal cycle is to raise and lower the piston at a constant temperature rather than at a constant pressure.  On the P-V diagram it would look like this (in green), overlayed against the isobaric cycle for comparison’s sake.

Assuming we have the same number of moles of particles in our diatomic gas again, n  = 1.2 mol…

(1) → (2)  Isothermal expansion:  For the isothermal cycle, we’ll start off with the gas as before: V1 = 0.001m3, and P1 = 6 Mpa as before.  And also Th = 600K.  Then we release the piston.  Instead of adding heat at a rate required to keep the pressure constant during the expansion, we add heat to keep the temperature of the gas constant at Th during the expansion.  Because the temperature is held constant, this expansion is called isothermal.  Eventually the piston will reach the top of its chamber, situation (2).  Here its volume will be V2 = 0.01m3.  And from the equation of state its pressure will be P2 = 0.6 Mpa.

(2) → (3)  Isochoric cooling: Just like before we need to cool the gas (at constant volume) to lower its pressure so that the piston will allow the wheel to continue rotating CW.  We will cool the gas to the temperature Tc = 300 K.  As such, from the equation of state, the pressure will be P3 = 0.3 Mpa.

(3) → (4)  Isothermal contraction:  Now that the gas has been cooled we will keep it at this termperature, Tc = 100K and allow the piston to drop back down until it arrives at its initial volume.  There its volume will be V4 = 0.01m3, and its pressure will be, from the equation of state, P4 = 3 Mpa.

(4) → (1)  Isochoric heating:  Now that the gas is at the end of its cycle, we need to prepare it to force the piston upwards again.  So at nearly constant volume, we will heat the gas back up to tempeature Th = 600 K to return it to its initial state.

At this point the cycle is complete and allow it to continue in order to keep raising the weight.  Our question will be again, what is the work done by our gas?  What is the heat input?  What is the heat output?  What is the change in energy of our gas?  What is the total change in entropy of our gas and the temperature baths?  What is the efficiency of our machine?  Is this an improvement on our previous cycle or not?

Net work done by gas

The net work is the sum of the works done during each of the 4 processes.

So we do less work in this cycle than in the previous one.  Now let’s see how much is input.

Heat input

What is the heat that we have to input?  We input heat during process (1) → (2), and during (4) → (1).  So the net heat input is:

We can determine the heat input during (1) → (2) using the 1st law of thermodynamics,

Note the change in energy during this process is 0 because the temperature isn’t changing.  And the work done by the gas is 10 kJ as we calculated before.  So,

And now let’s determine the heat input for process (4) → (1).  This is:

So, the net heat input is:

Observe that we have to input a lot less heat using this cycle than using the previous one.

What is change in energy of our gas?

This is easy to answer.  The change in energy of our gas is:

But note that the temperature of our gas doesn’t change from beginning to end of cycle.  This is because at beginning and end, it has the same P and V.  And since PV = NkT,  that means it has the same T as well.  So ΔT = 0 → ΔE = 0.  This is always the case for a cyclic process.

What is the heat output?

From the 1st law, we have,

Let’s apply this to the entire cycle.  Then we must have,

What is the change in entropy of our system?

Our system comprises the machine and the heat / cold sources that transfer heat to and from the machine.  This is a closed system since energy is transferred only between these objects.  And now we’d like to know the net change in entropy of this system.  Well,

The reasoning is this.  First, the net change in entropy of our gas is 0 because it goes through a cyclic process.  It begins and ends in the same state (T1, V1) and so it must have the same entropy at the end as it does in the beginning.  The ice bath increases in entropy because it accepts an amount of heat Qout = 14 kJ at a temperature of 300K (we make the standard simplifying assumption that the ice bath is so large that accepting that heat doesn’t change its temperature appreciably).  The fire decreases entropy because it gives an amount of heat Qin = 14 kJ away at temperature T = 600K.

What is the efficiency of our engine?

The efficiency of our engine is

So this is quite a bit more efficient than our previous cycle, over twice as efficient in fact.  So even though we do less work per cycle, overall we would have to use a lot less energy than in the 1st cycle to raise the weight to the top of the building.

So why is this cycle so much more efficient than the previous one?  The answer is that it wastes a lot less energy.  Remember that ΔS is a measure of how much waste of mechanical energy there is, and ΔS = 3.3 J/K for this cycle vs. 543 J/K during the other one.  The major savings in entropy in this cycle vs. the last was accomplished by replacing the isobaric expansion/compression with the isothermal expansion/compression.  The isobaric expansion/compression is an irreversible process because we have the fire/ice-bath at a higher/lower temperature than the gas and so heat will flow irreversibly from the fire to the gas, and from the gas to the ice bath because of the  temperature difference.  But heat flow during an isothermal expansion/compression doesn’t generate any net entropy, as alluded to in the previous lecture.  To demonstrate, consider the entropy change of the system during the isothermal expansion alone.  It is:

And the same holds for the isothermal compression.  So the only net systemic entropy loss occurs during the isochoric processes at the beginning and middle of the process.

So, can we do any better with another cycle?  We can in fact.  The key is to design a cycle that doesn’t waste any mechanical energy, in the entropy sense.  In other words we want a cycle for which the net entropy increase is 0.  According to the 2nd law of thermodynamics, this cycle must consist of purely reversible processes then.  The isothermal expansion/compression is reversible.  The only thing we need to change is the isochoric processes.  What we do is replace them with adiabatic processes (meaning during that process no heat is transferred to or from the gas).  Basically, instead of putting heat into and out of the gas to get its pressure to the required values for state (1) and (4), we let the gas naturally do this to itself by during +/- work on itself.  This cycle, with the maximum possible efficiency is called the Carnot cycle.

Method 3: Carnot cycle

The Carnot cycle is shown below, in red, overlaid against the previous two cycles.

(1) → (2) Isothermal expansion:  As before, we start the gas off at V1 = 0.001m3, P1 = 6Mpa, and Th = 600K.  And as before we let the piston start to expand isothermally.  But we stop the expansion at V = V2, and thereupon, let it expand adiabatically.  We will determine what V2 must be later.

(2) → (3)  Adiabatic cooling:   Now we want to cool the gas.  Instead of actively taking out heat, we let the gas lose heat itself by insulating it from the heat source.  As it continues to expand up to V3 = 0.01m3 it will do work on the piston.  This work must come out of its internal energy rather than the outside heat source this time.  Therefore its internal energy, and consequently, temperature, will drop.  Wben it gets to (3) it will have  V3 = 0.01m3, Tc = 300K, and consequenctly, P3 = 0.3 Mpa.

(3) → (4)  Isothermal contraction:  Now we allow the piston to descend by maintaining the gas at the constant temperature (which we can maintain by putting it back in contact with our cold reservoir), Tc, until it gets to the volume V4 (to be determined)

(4) → (1)  Adiabatic heating:  Then we need to heat the gas up to prepare it to push the piston back up again.  Instead of putting the heat in ourselves, we allow the gas to naturally heat up by insulating it from the cold reservoir.  Then, as the gas continues to contract under the weight of the piston, the piston will be doing work on the gas, which will go into the gas’s internal energy, heating it up until it gets to back to state (1) at temperature T = 600K, V = 0.01m3, P = 6 Mpa.

At this point we can start the cycle all over again and continue lifting the weight.  Again, we’d like to determine what work our engine performs on the weight, what heat input is necessary, and what heat ouput is necessary.  We want to know the change in energy of the gas, and the system’s total change in entropy.  Finally, we want to know the cycle’s  efficiency, which would be the maximum efficiency obtainable, for our given temperature reservoirs.

Work done by gas

The net work is the sum of the works done during each of the 4 processes.

We need to figure out what V2 and V4 are to proceed.  We also need to know what the pressure is during paths (2) → (3) and (4) → (1) as well.  To get V2 we use the fact that

for process (2) → (3), since it is an adiabatic path we must have,

and during (4) → (1) we must have,

where we have used that in the second and fourth process the pressure is given by the equation for adiabatic expansion/contraction equation: PVγ = const.  Now back to the work.  We have:

So we do even less work in this cycle than in the previous one.  Now let’s see how much heat is input.

Heat input

Heat is input during the isothermal expansion only (1) → (2).  From the 1st law, we have,

What is change in energy of our gas?

This is easy to answer.  The change in energy of our gas is:

But note that the temperature of our gas doesn’t change from beginning to end of cycle.  This is because at beginning and end, it has the same P and V.  And since PV = NkT,  that means it has the same T as well.  So ΔT = 0 → ΔE = 0.  This is always the case for a cyclic process.

What is the heat output?

From the 1st law, we have,

Let’s apply this to the entire cycle.  Then we must have,

What is the change in entropy of our system?

Our system comprises the machine and the heat / cold sources that transfer heat to and from the machine.  This is a closed system since energy is transferred only between these objects.  And now we’d like to know the net change in entropy of this system.  Well,

The reasoning is this.  First, the net change in entropy of our gas is 0 because it goes through a cyclic process.  It begins and ends in the same state (T1, V1) and so it must have the same entropy at the end as it does in the beginning.  The ice bath increases in entropy because it accepts an amount of heat Qout = 2 kJ at a temperature of 300K (we make the standard simplifying assumption that the ice bath is so large that accepting that heat doesn’t change its temperature appreciably).  The fire decreases entropy because it gives an amount of heat Qin = 4 kJ away at temperature T = 600K.  As we see, the net entropy change is 0, indicating this is as efficient a process as possible.

What is the efficiency of our engine?

The efficiency of our engine is

So the Carnot cycle is just a little more efficient than the isothermal cycle.

Maximum efficiency possible for a heat engine

As we argued, the Carnot engine is the most efficient possible.  But is it always 50% efficient, or what does the efficiency depend on?  Let’s idealize a heat engine and use the 1st and 2nd laws of thermodynamics to answer this question.  First, generally speaking a heat engine takes in some amount of heat QH from a hot reservoir at temperature TH, and releases some amount of heat QC to a cold reservoir at temperature TC, while doing some work W in the process – symbolized by the following diagram.

Then from the first law as applied to engine we have:

and from the second law applied to the engine + reservoirs, we have:

So our two equations are:

Solving for QH by multiplying the bottom equation by TC and adding the two equations together, we will get:

And we can plug this into our equation for the efficiency:

And this simplifies to:

So this tells us that non-equilibrium processes in our reservoir+engine system will lower the efficiency.  So we can maximize efficiency by using a cycle for which ΔSneq. = 0 (and this would be the Carnot cycle).  And then if this is the case, we have:

Observe that the efficiency goes to 1 as the separation between the temperatures goes to ∞.  Also, if we could operate an engine at absolute zero, then the efficiency would be 1.

Example

What is the efficiency of the Carnot engine in the example above?

The efficiency would be:

Clearly this formula is a little easier to use than calculatng the work, heat input seperately.