Refrigerators and Heat Pumps

Refrigerators

Now let’s consider a few examples pertaining to refrigerators.  Refrigerators are machines that are designed to do the opposite of heat engines.  Instead of absorbing heat and doing work with it.  They do work to take heat out of a place, and export it to another place.

For example’s sake we’ll say we have 82 moles of a diatomic gas (perhaps air again) initially at pressure P1 = 2Patm., and V1 = 1m3.  Suppose it is initially room temperature  T1 = Tc = 25˚C.  And we want it to extract heat from the room and export it to the outside, where Th = 38˚C.  Then a Carnot refrigeration cycle would look like this:

(1) → (2)  Isothermal expansion: We would insulate the gas from the outside, and put it in thermal equilibrium with the room  Then the AC unit will pull the piston upward, and allow the gas to expand in thermal equilibrium with the room so that the temperature of the gas holds constant at Tc = 25˚C.  As the gas expands, it will do work on the piston, and thereby draw an amount of heat Q from the room equal to the amount of work, W, it does.  The reason is this.  As the gas pushes the piston up a little bit, it will do a tiny bit of work, dW, on it.  As a consequence it would lose a tiny bit of internal energy dE = dW, and so its temperature would drop.  But as soon as its temperature drops, and amount of heat dQ = dE, from the room will conduct into the gas, restoring the gas’s temperature to Tc.   So in this way the gas will draw heat from the room equal to the amount of work that it does all the while maintaining its temperature at Tc.  This will take it to state (2), at volume V2 = 1.5m3 we’ll say.  Since its an isothermal expansion, T2 = 25˚C, and from the equation of state P2 = 1.35atm.

(2) → (3)  Adiabatic compression:  Now that we’ve absorbed an amount of heat Q from the room, we need to get rid of it – to expel it into the air.  But in order for heat to flow from the gas to the outside we need to raise the gas’s temperature to at least Th.  So what we do is this.  We insulate the gas so that no heat can flow out or into it, and the AC unit would compress the gas to a volume V3.  As we do so, we do work on the gas, which will increase its internal energy – i.e. its temperature, until it temperature comes to Th.  So we have T3 = 38˚C.  From the adiabatic equation T3V3γ-1 = T2V2γ-1, we can infer that V3 would be V3 = 1.35m3.  And from the equation of state we’d have P3 = 1.57atm.

(3) → (4) Isothermal compression:  Now that the gas is at outside temperature, we will want to expel the heat that we absorbed from the room.  So we put the gas in thermal contact with the outside.  To expel the heat Q that we absorbed from the room, the AC unit would push the piston downward.  We will do an amount of work W equal to the heat we absorbed in the first step, Q, and thereby push all of that heat out of the gas.  The reason this happens is this.  As we do a little bit of work, dW, on the gas, the gas would tend to gain an amount of energy dE = dW, and hence its temperature would tend to increase.  But as soon as its temperature increases in will be at a temperature greater than Th and heat dQ = dE will then naturally flow from the gas to the outside air.  In this way, all the work we do on the gas will result in the same amount of heat flowing out of the gas.  This will then bring us to the situation below (4).  The temperature of the gas will be T4 = 38˚C.  Its volume will be V4 = 0.9m3, and from the equation of state, P4 = 2.35atm.

(4) → (1) Adiabatic expansion:  Now we’re almost done with our cycle.  Next we need to lower the temperature of the gas to Tc so that it will be ready to accept more heat from the room.  So we insulate the gas on the AC unit will pull the piston upward, allowing the gas to expand.  As the gas expands, it does work against the piston, its internal energy and therefore temperature will drop – remember the gas is now insulated and so no heat can flow in to make up the difference.  We will allow the gas to expand just enough so that its temperature drops to Tc and we are back to our original situation, ready to start all over again (1).

At this point we could continue our cycle over again, and continue extracting heat from the room.  We can write out our cycle on a P-V diagram just like before.  It would look like this.

Process (1) → (2) is an isothermal expansion at Tc which draws in heat Q from room.  (2) → (3) is an adiabatic (i.e. fully insulated so no heat can be exchanged) compression which raises temperature of gas to Th.  (3) → (4) is an isothermal compression at Th which expels heat Q into the air.  And (4) → (1) is an adiabatic expansion which cools the gas back to temperature Tc.

In any event, as before, we would like to ask ourselves a few questions.  Namely, how much heat can we extract per cycle with our machine?  How much heat do we export to the air outside?  How much work does our machine have to do to take the heat out?  What is change in energy of our gas?  What is the net change in entropy?  How efficient is our machine at this heat extraction?

How much heat do we extract?

Between (1) → (2) we extract heat from the room.  We can use the 1st law to determine what that is.  We have,

So Q = W, where W is the work done by the gas.  So

How much work is done by the AC unit?

The total work done by AC unit is just negative the work done by the gas, which is just the area inside the P-V curve.  So we need to find the area inside the P-V curve.

During (1) → (2), from the 1st law we have,

During (2) → (3), from the 1st law we have,

We have assumed that we’re dealing with a diatomic gas again and so set f = 5.  And we have used the fact that this is an adiabatic process where no heat is exchanged.

During (3) → (4), from the 1st law we would have,

But we don’t know the heat, Q, given off during this process.  So we will have to calculate the work from the integral itself.  It is (remember the work done on the gas is negative the work done by the gas).

So now we can calculate the work.  It is:

Finally, between (4) → (1), from the first law, the work is:

So the net work is:

How much heat is output to the air?

Heat is output during process (3) → (4).  From the 1st law again we can determine that the heat output is:

So the heat output is just the work done on the gas during this process.  And this is:

What is the change in energy of our gas?

Since the gas begins and ends at the same state, the change in energy is 0.

What is the net change in entropy of our system?

The net change in entropy is 0 as well, since the cycle used is the Carnot cycle, which is entirely reversible, and which according to 2nd law must have no net change in entropy.

What is the ‘coefficient of performance’ of our refrigerator?

The coefficient of performance is defined as:

And so our coefficient of performance is:

Maximum possible performance coefficient of a refrigerator

Just as a Carnot engine is the most efficient engine possible, a Carnot refrigerator is the most efficient refrigerator possible.  Just like before, we’ll use the 1st and 2nd laws of thermodynamics to determine what this maximum efficiency is.  A schematic for our refrigerator is as follows,

Then from the first law as applied to refrigerator we have:

and from the second law applied to the refrigerator + reservoirs, we have:

Solving for QC we will get:

and so the coefficient of performance of our refrigerator would be:

multiplying top and bottom of that fraction by TC/TH, we can write it in more standard form,

and this tells us that non-equilibrium processes in our reservoir+refrigerator system will lower the performance.  So we can maximize performance by using a cycle for which ΔSneq. = 0 (and this, again, would be the (reverse) Carnot cycle).  And then if this is the case, we have:

Example

What is the performance coefficient of the Carnot refrigerator in the previous example?

This would be:

which is close enough to what we had before (rounding error)

Heat Pumps

A refrigerator removes heat from a cold environment and adds heat to the hot environment.  We call this device a refrigerator because we are interested in its ability to remove heat from the cold environment, thereby making it colder.  But if we were interested in the refrigerator’s concomitant property of adding heat to the hot environment – thereby making it hotter – then a more apt appellation for the refrigerator would be: heat pump.  And a measure of the heat pump’s performance would be the ratio of heat added to the hot place / work done on machine.  So we’d redefine κ in this context to be:

If we run through the calculations we did for the refrigerator, we’d find that we can also write κ as:

and the maximum coefficient of performance would be for one which produces 0 entropy over the cycle, as usual.  Setting ΔSneq. = 0, then, we’d find:

This is just 1 over the Carnot engine efficiency, which makes sense because κ = 1/η.