Transformers

Wall outlets in Europe supply power at 220V (rms), while here in the US they supply power at 120V.  Appliances manufactured for the US are designed with the 120V source in mind, and would often blow if connected directly to a 220V outlet.  We can insert what’s called a transformer into the line to step down the potential (or up if we wanted).

Taking a more abstract perspective.  We’ve learned how to find currents and potentials when we put resistors into a circuit in series, or in parallel.  Another way to connect resistors to circuits is to connect them via transformers.  We now want to study how to calculate the ‘equivalent resistance’ of a resistor connected via transformer, and then we also want to determine how to calculate I, and ΔV across circuit elements as well.  To that end, let’s consider the following circuit,

Clearly Ip would be Vrms/Rp = 5A.  Now what happens if we add a resistor, say 250Ω, into the circuit via a transformer?  What would be the new current Ip?  This would look like,

Rs is connected to the circuit containing Vrms and Rp via the box.  The box is a ‘transformer’.  The blue curly things are inductors (usually solenoids of some sort) and are usually called ‘coils’.  Np is the number of turns in the ‘primary’ coil.  Ns is the number of turns in the ‘secondary’ coil.  For future reference, assume Np = 100, and Ns = 10.  Vp and Vs are the potential differences across the respective coils.  Ip and Is are the respective currents.  Now the battery is driving current through the primary coil.  This current will set up a magnetic field through the primary (as we discussed before).  Since the current is changing, the flux will change, and by Faradays law will set up a back emf across the primary coil.  Additionally, however, the field lines will be guided into the secondary coil, and the fact that they’re changing will induce an emf in the secondary coil.  Since this will drive current in the secondary coil, a magnetic field in the secondary will be set up and these field lines, changing as they are, will create a back emf in the secondary coil.  Furthermore, these field lines will be guided into the primary, inducing another emf in there as well.

In any event, call the flux penetrating each loop of the primary Φ.  Then the total flux going through the primary coil is: NpΦ.  Since the current is changing, the flux is changing, and now by Faraday’s law, the potential difference across the primary coil is:

which implies that the rate of change of flux would be,

Now this flux, and rate of change of it is preserved in the secondary because the transformer usually contains an iron core which guides the B filed lines in the primary into the secondary.  Therefore the total flux going through the secondary will be NsΦ.  And by Faraday’s law, we have the potential difference across the secondary to be:

Solving for dΦ/dt we get,

Equating the two expressions for dΦ/dt we obtain,

We can obtain another equation from energy conservation.  The rate that energy is being output by the battery is equal to the rate it is being dissipated by the two resistors (one in primary, one in secondary).  But the rate at which power is being dissipated by the secondary resistor must be equal to rate at which power is being generated across the secondary coil, and this in turn must then be equal to the rate at which power is being generated across the primary coil.  So we have that Pp = Ps, i.e.:

And this requires:

Now, let’s determine the ‘equivalent resistance’ of transformer & Rs in the primary circuit.  The equivalent resistance would be defined as:

So,

χp is the ‘equivalent resistance’ of Rs as seen by the battery (this is called Zp in the book).  And we’d be able to represent our equivalent circuit as:

where the box is the pictorial representation of the equivalent resistance of the transformer & Rs.  Now to calculate the current, we need to know the equivalent resistance of χp and Rp.  Now, it is rather complicated to determine the equivalent resistance of  χp and Rp – remember we usually cannot just add the two together when we have AC circuits.  But it turns out that if Rs is smallish, then the equivalent resistance of Rp and χp is approximately Rp + χp.  This can be certified by using energy conservation arguments.  Going around the two loops, and applying energy conservation, we have:

Plugging in we have:

So we have:

Going back to our example, then,

Therefore Ip = 2.5kV/25.5kΩ ≈ 0.1A.  By the way, a nice way to remember the ratio of turns is this:

The ratios follow the arrows, from front to back.  Vp = (Np/Ns)Vs, and Vs = (Ns/Np)Vp.  Same goes for Req. and Rs.  When bringing Rs up into the primary, the equivalent is Req. = (Np/Ns)2Rs.

Example 1

Consider you’re in France and you need a transformer to reduce the output voltage from 220V to 120V so that your laptop will not break.  What should be the ratio of turns in the primary to the secondary?  The set up looks like this:

Since you want to lower the voltage, you want the number of turns in the secondary to be lower than in the primary.  So it should look like above.  And specifically,

Now, if the resistance of your laptop is 100Ω, what is the current in the primary and in the secondary?   To find the current in the primary, we can determine the equivalent resistance of the transformer & laptop.  We have,

So then we can replace the transformer & Rlaptop with their equivalent resistance,

Therefore the current in the primary is:

What is the current in the secondary?  Well we can go back to:

and replace Vp = 220 with its equivalent in the secondary coil.  The transformer steps the down the potential Vrms to:

,

and so we can represent the circuit as,

since we said the resistance of the laptop was 100Ω, the current will be:

Example 2

The main use of transformers is actually for reducing power loss when transmitting electricity over long distances.  Consider the following example,

OK, then what is the power supplied by the battery?  This is:

And what is the power lost through the wires?

So nearly all of the power that is supplied by the power station is lost by thermal dissipation.  The fraction is 395/444 ≈ 89%.

That is not good since it costs money to generate electricity.  How can we solve this problem?  Use a transformer to step up the voltage and thereby lower the current, minimizing power loss.  Consider the following circuit.

With the first transformer we step up the voltage, and with the second we step it back down.  Let’s suppose N2 = 100 and N1 = 10.  Then what is the power loss in the lines?  Well, the equivalent resistance of the transformer & RH is,

This makes our circuit,

Now the transformer steps up the potential difference across the primary into the secondary.  Let’s determine the potential difference in the secondary,

So we can write this as:

Now what is the current in this circuit?  We can use,

To say that

Therefore the current is:

The power supplied by the battery is equal to the total power lost.

The power lost through the wire resistors is:

And so the fraction of power lost is 27/370 ≈ 7% - much lower than before.

Appendix

Let’s reprise our transformer analysis using a little more rigour.  Consider our archetypal example:

Let’s write out KVL in the top loop.  Well first, we need an expression for the back emf in the primary in terms of the currents in the primary and secondary coils.  We have:

Now let’s work out the NpLs/Ns term.  We’ll presume solenoids with inductances L = ℓμ0n2A.  We’ll also assume (we already have I guess) that all the flux going out of one solenoid goes into the other – so they are perfectly coupled.  So then let’s take the ratio of inductances.

I think one typically presumes the same geometry, but just different winding densities.  Then,

and we can write:

This is usually written:

where M is defined as the mutual inductance.  More generally M = k√(LpLs) where k runs between 0 and 1.  So then our two KVL equations are as follows:

and taking the Fourier transform of these equations (assuming, e.g., V(t) = Ve-jωt and i(t)  = ie-jωt, where j = imaginary unit), we have:

Can rewrite as:

Then use Cramer’s rule,

We can get ip too.

Assuming mutual inductance for ideal transformer is M = √LpLs:

If frequencies are large, specifically if:

Well, Rs/Ls is larger than 1/(Ls/Rs + Lp/Rp) anyway, so I guess we can just require:

then we have:

Since Lp/Ls = Np2/Ns2, we see our results are purely turn-dependent, and conform to the equivalent resistance formula we heuristically derived above, namely:

We also see the relationship:

which we derived above too.  For the sake of comparison, let’s say that in our example above Lp = 30mH and Ls = 0.3mH.  And assume f = 60Hz.  What would ip be?  Well,

The rms of these currents is basically:

And compare to our approximate formula:

So our approximations are waaay off.  If the frequency were f = 60kHz, then our approximate formulas would be much better approximations.