Combinations of Resistors

Now we want to combine resistors and see if we can work out how much current each one will get.

Finding currents in a network of resistors via equivalent resistance

If we connect a single resistor to a battery, we can immediately ascertain the current, because we would know the potential difference across the resistor (just that of the battery).  But what if we have something like this:

To make progress on such a circuit, we can use a method of simplifying the circuit by recognizing so-called series and parallel combinations of resistors.  Circuit elements (capacitors, resistors, etc.) are in series if there is a path between them that doesn’t cross a circuit junction.  So for instance R3 and R5 are in series (none of the others are).  Circuit elements are in parallel if you can draw a complete loop through them w/o crossing any other circuit elements.  And so R4 and R2 are in parallel, as are R6 and R1 (but no others).

Resistors in series all carry the same current, and as well have the same current as their equivalent resistor Rseries.   The equivalent resistance of series resistors is defined as the single resistor which would draw the same current from the rest of the circuit:

Adding up potential differences around the left circuit we’d have ΔV – IR1 – IR2 – IR3 – IR4 – IR5 = 0 → I = ΔV/(R1 + R2 + R3 + R4 + R5).  In the equivalent resistance circuit we have I = ΔV/Req..  Since these are equal, by the definition of the equivalent resistance, we must have Req. = R1 + R2 + R3 + R4 + R5.  And in general we’ll have:

Resistors in parallel each have the same potential, as well as the same potential as their equivalent resistor Rparallel.  Again, the equivalent resistance is defined as the resistance of the single resistor that would draw the same current from the rest of the circuit:

and we can work out what this would be.  From current conservation we must have I = I1 + I2 + I3 + I4 + I5 = ΔV/R1 + ΔV/R2 + ΔV/R3 + ΔV/R4 + ΔV/R5 = ΔV(R1-1 + R2-1 + R3-1 + R4-1 + R5-1).  And from the definition of equivalent resistance we must also have I = ΔV/Req..  It follows then that Req-1 = R1-1 + R2-1 + R3-1 + R4-1 + R5-1.  Or to put it another way:

We can often use these facts to work out the currents in a circuit containing multiple resistors.  For instance, the circuit above would be reduced as follows:

Example

What is the currrent through the battery?  Let ΔV = 10V, and R1 = 1Ω and R2 = 2Ω.

The resistors are in series so Req. = R1 + R2 = 3Ω.  So the current through the battery is:

This is also the current through each individual resistor.

Example

What is the current through the battery?  and through each individual resitor?  Let ΔV = 10V, and R1 = 1Ω and R2 = 2Ω.

The equivalent resistance is:

Therefore the current through the battery is:

The current through R1 is:

and the current through R2 is:

Note that ΔV1 and ΔV2 = 10V, the potential difference across the battery, because they are connected directly across the battery.  Also note that the current going through the battery splits up into I1 and I2, so naturally I1 + I2 = 15A, the current through the battery.

Example

Consider the following example,

Calculate the equivalent resistance, current flowing through the battery, current flowing through each of the branches.  Total power dissipated.  Let ΔV = 10V, and let R1 = 1Ω, R2 = 2Ω.  Then we’ll start by combining bottom R1 and R2 in series,

Now R2 and 3 are in parallel, so,

So we have,

and now these are in series.  Req = 1.2 + 1 = 2.2Ω, so,

So the equivalent resistance of the circuit is 2.2Ω.  Therefore the current through the battery is: I = ΔV/R = 10/2.2 = 4.55A.

And the total power supplied by the battery is therefore,

The total power dissipated by the circuit is:

Of course they must be the same because of energy conservation.  Now, let us ask what the current is in the circled resistor.

Well, we work backwards like with capacitors.  Starting from the equivalent circuit,

We expand the 2.2 in series to get the 1.2 and the 1.  Since we expanded in series, the 2.2 equivalent, and the 1.2, 1 constituents all have the same current.  Therefore, we have,

Now, we expanded the 1.2 in parallel to get R2 and the 3,

Since we expanded in parallel, it is the potential difference which will remain the same between the 1.2, and its R2 and 3 constituents.  We must therefore calculate the potential difference across the 1.2.  By Ohm’s law is is ΔV = IR = (4.55)(1.2) = 5.46.  Therefore we have:

Now it was R2 for which we were trying to find the current.  Since we know ΔV and R, we use Ohm’s law to conclude,

So we have the current through R2 is 2.73A, illustrated below,

which is what we wanted to know.  We could calculate the power dissipated across R2.  This would be,